2022-2023学年第二学期《液压与气动传动》期末考试

# B卷答案及评分标准

# 一、Fill in the blanks. (20 points, 1 point for each blank)

1．atmosphere pressure   absolute pressure

2.  vapour pressure

3.  gear pump  vane pump  piston pump

4．accumulator   filter   reservoir  piping (or seal)

5．Equation of continuity (conservation of mass)   Bernoulli equation (conservation of energy)  Equation of momentum (conservation of momentum)

6．laminar   turbulent

7．hydraulic energy   mechanical energy

8．increase

9．flow rate  external load

# 二、Short answer question items. (30 points, 6 points for each)

# 1. (1) power input segment. (2 points)

# (2) the power output segment. (1 points)

# (3) control element. (1 points)

# (4) assistant element. (1 points)

# (5) working oil. (1 points)

# 2. (1) In order to make the gear pump can supply oil continuously and smoothly,the overlap coefficient ε of gear mesh must be greater than 1.In this way,the gear in the meshing process,the first pair of gear teeth have not yet out of engagement,after a pair of gear teeth have entered engagement. Because the two pairs of gear teeth mesh at the same time,there is a part of the oil trapped in the two pairs of gear teeth formed by the independent closed chamber,this closed chamber and the pump suction,pressure oil chamber is not connected to each other.When the gear rotates,the volume of the closed chamber changes, so the oil is compressed or expanded,this phenomenon is called the phenomenon of surrounded oil of the gear pump. (3 points)

# (2) When the volume of the closed chamber is reduced, the trapped oil is squeezed, resulting in high pressure,so that the shaft and bearing are subjected to great impact load;When the volume of the closed cavity increases,local vacuum is formed,which is easy to produce cavitation phenomenon, and the pump produces vibration,noise and cavitation phenomenon. (1 points)

# (3) The way to eliminate the phenomenon of surrounded oil is to open the unloading groove on the end cover on both sides. (2 points)

# 3. The basic conditions that make up a hydraulic pump are:

# 1) Oil tight chamber. The size of this oil tight chamber will change periodically. (2 points)

# 2) Flow deploying. The corresponding mechanism is used to ensure the inlet port is separate from outlet port. (2 points)

# 3) The absolute oil pressure in the reservoir should be equal or more than the atmospheric pressure which is absolutely necessary external condition for a capacity pump. (2 points)

# 4. Differences:

# 1) the pressure reducing valve holds the outlet pressure constant, whereas the relief valve holds the desired reduced pressure at the inlet. (2 points)

# 2) the pressure reducing valve is normally open while the relief valve is normally closed. (2 points)

# 3) the pressure exists in both inlet and outlet ports of pressure reducing valves, so the leaking oil from the spring port of pilot valve must induced separately to the reservoir. But the outlet pressure oil of relief valve flows directly into the reservoir and the leaking oil from the spring port of pilot valve discharge out through the inner passages of valve body. (2 points)

# 5. (1) Three types of speed control circuits:

# 1)Throttle speed-regulating circuit. (1 points)

# 2) Volume speed-regulating circuit. (1 points)

# 3) Volume-throttle speed-regulating circuit. (1 points)

# (2) Three types of throttle speed regulating circuits:

# 1) Inlet throttle speed regulating circuits. (1 points)

# 2) Exit throttle speed regulating circuits. (1 points)

# 3) Bypass throttle speed regulating circuits. (1 points)

# 三、Calculations. (20 points, 10 points for each)

- 1.

- （1）It can be obtained from the liquid flow equation:

(2 points)

(1 point)

（2）It can be obtained from the continuity equation of liquid flow:

(1 point)

So:                               (1 point)

So:               (1 point)

- If the Bernoulli equation is written in sections 1-1 and 2-2, then：

- (2 points)

- So: the pressure difference between section 1-1 and section 2-2：

- (1 point)

- So:         (1 point)

- 2.

- （a）Thrust：    (1 point)   Speed：  (1 point)

- Movement direction: hydraulic cylinder to the left    (1 point)

- （b）Thrust：     (1.5 points)   Speed：     (1.5 points)

- Movement direction: hydraulic cylinder to the right     (1 point)

- （c）Thrust：     (1 point)   Speed： (1 point)

- Movement direction: hydraulic cylinder to the right     (1 point)

IV Diagramming question. (15 points)

- (1) Inlet throttle speed-regulating circuit.(3 point)

- (2) Locked circuit.(3 point)

- (3) Pressure-increasing circuit.(3 point)

- 1-pump; 2-relief valve; 3-cylinder; 4-bidirectional hydraulic lock; 5-Three-position, four-way directional control valve; 6-intensifier.(1 point for each)

V Analysis. (15 Points)

- (1) When the left solenoid of the two-position，five-way solenoid operated directional control valve 8 is energized,the hydraulic cylinder is supplied with oil by the large flow pump 2 and performs differential fast feed.At this point,the oil inlet is:hydraulic pump 2two-position，five-way solenoid operated directional control valve 6 (right position)two-position，five-way solenoid operated directional control valve 8 (left position)hydraulic cylinder 9 left chamber.The return oil circuit is:hydraulic cylinder 9 right chambertwo-position，five-way solenoid operated directional control valve 8 (left position)two-position，five-way solenoid operated directional control valve 6 (right position)two-position，five-way solenoid operated directional control valve 8 (left position)hydraulic cylinder 9 left chamber.(6 point)

- The basic circuit is a differential area connection circuit. (2 point)

- (2) Turn on the left solenoid of the two-position,five-way solenoid operated directional control valve 6,and turn off the left solenoid of the two-position，five-way solenoid operated directional control valve 8.At this time,the oil circuit from the large flow pump 2 to the hydraulic cylinder 9 is cut off,while the small flow pump 1 and the speed control valve 5 are connected to the circuit,and the system enters the working feed. At this point, the oil inlet is:small flow pump 1speed control valve 5two-position，five-way solenoid operated directional control valve 6 (left position)check valve 7two-position，five-way solenoid operated directional control valve 8 (right position)hydraulic cylinder 9 left chamber.The return oil circuit is:hydraulic cylinder 9 right chamber two-position，five-way solenoid operated directional control valve 8 (right position)two-position five-way solenoid operated directional control valve 6 (left position)oil tank.(5 point)

- The basic circuit used is the inlet throttle speed-regulating circuit.(2 point)