I Fill in the blanks. (20 points, 1 point for each blank)

- The hydraulic system can be divided into five logical segments, they are         ,         ,         ,          and working oil.

- Please write out three hydraulic auxiliary components:        ,         ,         .

- The three basic motion equations to describe the dynamics laws in flowing fluid are         ,         ,         .

- According to Reynolds test, the main states of fluid flow are          and          in a pipe.

- There are three basic types for speed control circuits. They are         ,         ,         . (Hints: Speed regulating can be done by two methods, i.e., changing input flow rate or displacement)

- In a pipe system, there are two energy loss types,          and         . (Hints: The first one is the energy losses due to viscosity in equal diameter pipe. The second one is the energy losses when fluid flows through pipe entrance or exit, sudden variety across-sections etc.)

- The common pressure operated valves include         ,         ,         , and pressure relay.

1．power input segment   power output segment   control elements   assistant element

2．accumulator   filter   reservoir (piping or seals)

3．Equation of continuity (conservation of mass)   Bernoulli equation (conservation of energy)  Equation of momentum (conservation of momentum)

4．laminar   turbulent

5. Throttle speed-regulating circuit, volume speed-regulating circuit, volume-throttle speed-regulating circuit

6. losses along circle parallel pipe, minor losses in pipe system

7. relief valves, pressure-reducing valves, sequence valves

II Short answer question items. (20 points, 5 points for each)

- Please explain the phenomenon of surrounded oil and the method for solving it.

Solution:

Phenomenon of surrounded oil: An oil tight chamber is formed between the two pairs of teeth when they mesh at the same time, As the gears rotate, the size of the chamber constantly changes from zero to the maximum and then back to zero during one complete revolution. As the size begins to increase, a vacuum is formed without oil compensation , which will cause cavitation and noise. As the gears continue to rotate , the size of the chamber decreases and the oil is forced out through the gap, which not only increases pressure and makes the gear bearing undertake periodic pressure pulsation, but also increases the temperature of the oil. The action described above is called phenomenon of surrounded oil. (3 points)

Load relief measure: Most gear pumps are designed with relief grooves on the front and rear cover boards or floating bearing sets ( floating sideboard). The grooves can minimize the size of the oil tight chamber (the chamber connects with the out let port when the volume of the chamber changes from the maximum to zero, and with the inlet port when it changes back from zero to the maximum). (2 points)

- What is the delivery of hydraulic motor? Please explain the volumetric efficiency and mechanical efficiency of hydraulic motor?

Solution:

The delivery of hydraulic motor is defined as the needed oil volume per revolution of the motor output shaft without oil leakage when the volumetric efficiency equals 1. (3 points)

Mechanical efficiencies: If we define the TMo as the actual output torque after startup and △TMo the torque loss at the dynamic state. However, the practical output torque TM is also less than the theoretical torque TMt due to the energy loss produced by the relative movement of each component and the relative movement between the fluid and the components, thus

TMo = TMt - △TMo

The mechanical efficiency of the motor, noted as ηMm

ηMm = TMo/ TMt = (TMt - △TMo)/TMt

Volumetric efficiency: The volumetric efficiency ηMV is the ratio of theoretical flow rate qMt to the practical flow rate qM

- The basic conditions that make up a hydraulic pump.

Solution:

1) Oil tight chamber. The size of this oil tight chamber will change periodically. (1 point)

2) Flow deploying. The corresponding mechanism is used to ensure the inlet port is separate from outlet port. (2 point)

3) The absolute oil pressure in the reservoir should be equal or more than the atmospheric pressure which is absolutely necessary external condition for a capacity pump. (2 points)

- Please write out the differences between pilot operated relief valves and pilot operated pressure reducing valves.

Solution:

1) the pressure reducing valve holds the outlet pressure constant, whereas the relief valve holds the desired reduced pressure at the inlet. (1 point)

2) the pressure reducing valve is normally open while the relief valve is normally closed. (1 point)

3) the pressure exists in both inlet and outlet ports of pressure reducing valves, so the leaking oil from the spring port of pilot valve must induced separately to the reservoir. But the outlet pressure oil of relief valve flows directly into the reservoir and the leaking oil from the spring port of pilot valve discharge out through the inner passages of valve body. (3 points)

III Calculations. (25 points)

1. In the horizontal pipe shown in the diagram below, an ideal fluid flows to the right, with its outlet open to the atmosphere. v1=3m/s, d1=80mm, d2=50mm. Assume that α is 1 and liquid density is 900kg/m3. Ignore all energy losses. Please calculate the parameters below. (10 points)

(1) Flow rate q through the cross-section 1-1. (3 points)

(2) The flow velocity v2 at the outlet. (3 points)

(3) The pressure difference Δp between the cross-section 1-1 and 2-2. (4 points)

Solution:

（1）Flow rate calculation：

(2分)

(1分)

（2）Continuity equation：

(1分)

then：                                       (1分)

so：                      (1分)

（3）Bernoulli equation at the cross-section 1-1 and 2-2:

(2分)

Pressure difference between the cross-section 1-1 and 2-2

(1分)

Thus：             (1分)

2. Consider the three different structural types of hydraulic cylinders shown in the diagram, with piston diameters D and rod diameters d. Given a flow rate q and pressure p into the cylinders, analyze the force generated, the velocity of the piston, and the movement direction for each type of cylinder. (10 points)

# Solution

（a）Thrust   (1分) velocity： (1分) movement direction：The cylinder body moves to the left.  (1分)

（b）Thrust     (1.5分)  velocity：    (1分) movement direction：The cylinder body moves to the right.    (1分)

（c）Thrust      (1分) velocity：    (1.5分)   movement direction：The cylinder body moves to the right.  (1分)

3. In the circuit shown in the diagram, the area of the cylinder chamber without rod is 50 cm², and the load FL is 10,000 N. The set pressures for each valve are indicated in the diagram. Calculate and determine:

(1) the pressures at points A and B when the piston is in motion; (3 points)

(2) the pressures at points A and B when the piston stops at the end of its stroke. (2 points)

Solution:

(1)Force analysis:

At this time, the pressure at point B has not reached the set pressure of the pressure reducing valve, causing the valve to be fully open, allowing free flow between the inlet and outlet. As a result, the pressures at points A and B are equal, with pA=pB=2 MPa.

(2)When the piston stops at the end position, the outlet of the pressure reducing valve no longer outputs oil, and its outlet pressure (pB) rises from 2 MPa to the set pressure of 3 MPa, which it maintains consistently.

At this time, the pressure reducing valve is in operation. Since only a small amount of oil leaks through the pressure reducing valve back to the tank, the hydraulic oil output from the pump can only return to the tank by opening the relief valve. Therefore, the pressure at point A should equal the set pressure of the relief valve, which is pA=5 MPa.

IV Diagramming question. (20 points)

Please write the name of basic hydraulic circuits (8 points) and hydraulic components (12 points).

（1）                                           （2）

C

B

A

A

C

B

| Hydraulic basic circuit |  |  | Hydraulic basic circuit |  |
| --- | --- | --- | --- | --- |
| Component A |  |  | Component A |  |
| Component B |  |  | Component B |  |
| Component C |  |  | Component C |  |

| Hydraulic basic circuit |  |
| --- | --- |
| Component A |  |
| Component B |  |
| Component C |  |

Solution:

（1）

| Hydraulic basic circuit | Sequence action circuit |
| --- | --- |
| Component A | Non-return valve (Check valve) |
| Component B | Sequence valve |
| Component C | Three-position, four-way directional control valve |

（2）

| Hydraulic basic circuit | Bypass throttle speed-regulating circuit |
| --- | --- |
| Component A | Hydraulic cylinder |
| Component B | Throttle valve |
| Component C | Reservoir |

(3)

| Hydraulic basic circuit | Pressure-increasing circuit |
| --- | --- |
| Component A | Single action intensifier |
| Component B | Invariable displacement pump |
| Component C | Relief valve |

V Analysis. (20 Points)

In the hydraulic system shown in the diagram, Pump 1 is a pump with a small flow rate and Pump 2 is a large-flow-rate pump. Actuator 9 finishes autocratically in a cycle motion of ‘fast extends → slow extends → quick retract’ respectively. Analyze the diagram and answer the following questions:

(1)How does the system achieve the ‘fast extends’ motion, what is the path of oil flow during this motion, and what is the name of the basic circuits used? (7 points)

(2) How does the system achieve the ‘slow extends’ motion, what is the path of oil flow during this motion, and what is the name of the basic circuit used? (7 points)

(3) How does the system achieve the ‘quick retract’ motion, and what is the path of oil flow during this motion? (6 points)

Solution

(1)When the left electromagnet of the two-position five-ways solenoid operated valve 8 is activated, the hydraulic cylinder is only supplied by the large-flow-rate pump 2, performing a fast extend via a differential connection.

The oil inlet path is: Hydraulic Pump 2  two-position five-ways solenoid operated valve 6 (right envelop)  two-position five-ways solenoid operated valve 8 (left envelop)  left chamber of Hydraulic Cylinder 9.

The return path is: right chamber of Hydraulic Cylinder 9  two-position five-ways solenoid operated valve 8 (left envelop)  two-position five-ways solenoid operated valve 6 (right envelop)  left chamber of Hydraulic Cylinder 9.

The basic circuit employed is a differential connection and fast-speed movement circuit by double pumps oil supply.

(2)When the left electromagnet of the two-position five-ways solenoid operated valve 6 is activated, and the left electromagnet of the two-position five-ways solenoid operated valve 8 is inactivated, the large-flow-rate pump 2 disconnects the left chamber of the cylinder 9. The small-flow-rate pump 1 and the flow control valve 5 are integrated into the circuit, initiating the ‘slow extend’ motion.

The oil inlet path is: The small-flow-rate pump 1  flow control valve 5  the two-position five-ways solenoid operated valve 6 (left envelop)  check valve 7  two-position five-ways solenoid operated valve 8 (right envelop)  right chamber of cylinder 9.

The return path is: right chamber of cylinder 9  two-position five-ways solenoid operated valve 8 (right envelop)  two-position five-ways solenoid operated valve 6 (left envelop)  reservoir

The basic circuit employed is inlet throttle orifice speed-regulating circuit.

(3)When the left electromagnet of the two-position five-ways solenoid operated valve 6 and 8 are activated, the hydraulic cylinder is only supplied by the large-flow-rate pump 2, performing a ‘quick retract’ motion.

The oil inlet path is: pump 2  two-position five-ways solenoid operated valve 8 (left envelop)  right chamber of cylinder

The return path is: left chamber of cylinder  two-position five-ways solenoid operated valve 8 (left envelop)  two-position five-ways solenoid operated valve 6 (left envelop)  reservoir